# General Purpose Sequencer Runtime Requirements

# 1. Purpose

The General Purpose Sequencer Runtime shall provide configurable automated sequencing for industrial and laboratory systems.

The system shall support:

- Runtime sequencing

- Hardware orchestration

- Interlocks

- Logging

- Parameterized execution

- Field-modifiable scripts

The system shall allow sequencing behavior to be modified in the field without requiring a software development environment.

# 2. System Architecture

## 2.1 Major Components

The system shall consist of:

Browser UI
Web Server
DB
Sequencer Runtime
Hardware Controllers

## 2.2 Responsibilities

Browser UI

Responsible for:

- Script selection

- Parameter editing

- Runtime status display

- Historical data display

- Administrative interaction

Web Server

Responsible for:

- Script administration

- Parameter administration

- Historical data access

- User interface data services

- DB access for UI operations

The Browser UI shall not directly access the DB.

Sequencer Runtime

Responsible for:

- Script execution

- Runtime state management

- Hardware orchestration

- Interlocks

- Logging

- Runtime parameter management

- Safe-state enforcement

The Sequencer Runtime shall access the DB directly for runtime operations.

The Sequencer Runtime shall not function as a general DB gateway for the UI.

Scripts

Scripts shall:

- Define sequencing behavior

- Read runtime parameters

- Write runtime parameters

- Request runtime operations through the approved system API

Scripts shall not:

- Access the DB directly

- Access the filesystem

- Access the network

- Launch processes

- Directly access hardware

# 3. Sequencer Runtime

## 3.1 Platform

The Sequencer Runtime shall:

- Execute on Linux

- Support self-contained deployment

- Support ARM64 deployment

## 3.2 Runtime Authority

Only the Sequencer Runtime shall:

- Command hardware during automatic operation

- Apply interlocks

- Control runtime sequencing

- Determine safe-state behavior

## 3.3 Automatic Operation

Only one top-level script may actively control the runtime at a time.

When a script is active:

- Automatic control mode shall be enabled

When the script completes, cancels, aborts, or faults:

- Automatic control behavior shall follow configured runtime policy

# 4. Script System

## 4.1 Script Language

The scripting language shall be JavaScript.

Scripts shall be stored in the DB.

## 4.2 Script Administration

The system shall provide:

- Create script

- Modify script

- Delete script

- Enable/disable script

through a web-based administrative interface.

## 4.3 Script Execution

The UI shall:

- Request the list of available scripts

- Allow the user to select a script

- Request script execution through the Sequencer Runtime API

The Sequencer Runtime shall:

- Load the selected script from the DB

- Validate the script

- Execute the script

## 4.4 Script Validation

Validation shall include:

- JavaScript syntax validation

- Hardware reference validation

- Parameter validation

- Timing validation

## 4.5 Hardware Reference Validation

The Sequencer Runtime shall validate that referenced hardware resources exist and are properly configured before script execution.

Examples include:

- Relay outputs

- Analog outputs

- Analog inputs

- Digital inputs

- Runtime parameters

## 4.6 Timing Validation

All timing values shall:

- Be expressed in seconds

- Be valid floating point values

## 4.7 Script Fault Policy

Programming or configuration violations shall immediately terminate script execution.

Examples include:

- Invalid hardware references

- Invalid parameter references

- Invalid timing values

- Forbidden runtime operations

The Sequencer Runtime shall:

- Log the fault

- Terminate the script

- Apply configured safe-state behavior

- Transition to FAULT state

Control shall not be returned to a misbehaving script.

# 5. Sequencer Runtime API

## 5.1 Runtime Operations

The Sequencer Runtime API shall support:

- Start script

- Cancel script

- Abort script

- Get runtime status

- Get active script

- Get fault state

## 5.2 Cancel Behavior

Cancel shall:

- Request cooperative script cancellation

- Allow the script to unwind execution

- Allow the script to place the system into a safe state

## 5.3 Abort Behavior

Abort shall:

- Immediately terminate script execution

- Immediately apply configured safe-state behavior

The runtime shall not rely on script cleanup during abort operations.

# 6. Script API

## 6.1 Script Interface

Scripts shall interact with the runtime through:

system.*

## 6.2 Script Functions

The system API shall support functions including:

system.setRelay()

system.setOutput()
system.setAnalogOut()

system.wait()
system.waitUntil()

system.getParameter()
system.setParameter()

system.setInterlock()
system.clearInterlock()

system.log()

system.runScript()

Additional functions may be added in future revisions.

## 6.3 Script Runtime Restrictions

The JS runtime shall not expose:

- Filesystem access

- Network access

- Process launching

- Reflection

- Arbitrary host runtime access

- Direct hardware access

# 7. Parameters

## 7.1 Parameter System

Parameters shall:

- Exist at runtime

- Be readable by scripts

- Be writable by scripts

- Be readable by the UI

- Be writable by the UI where permitted

## 7.2 Parameter Logging

Parameters may optionally be flagged for logging.

Logged parameters shall:

- Be written into the historical data stream

- Be timestamped

- Be queryable

- Be chartable

- Be exportable

Logged parameters shall behave similarly to logged hardware values.

## 7.3 Parameter Definitions

Parameters shall support:

- Name

- Data type

- Default value

- Min/max range

- Runtime editable flag

- Logging enable flag

# 8. Logging

## 8.1 Logging Requirements

The system shall log:

- Script execution

- Runtime events

- Faults

- State changes

- Hardware values

- Parameter values

- User actions

## 8.2 Script Logging

The system shall log:

- Script start

- Script completion

- Script cancel

- Script abort

- Script fault

- Major runtime actions

# 9. Safe-State Behavior

The system shall support configurable safe-state behavior for:

- Normal completion

- Cancel

- Abort

- Fault

- Lost hardware communication

- Application restart

# 10. Future Expansion

The architecture shall support future:

- Additional script functions

- Additional hardware modules

- Additional runtime modules

- Additional sequencing capabilities

- Additional logging capabilities

# 11. .NET 8 Pluggable Module Requirements

Purpose

The Sequencer shall support dynamically loaded .NET 8 assemblies that provide hardware or external-system access through a common interface.

Requirements

- The Sequencer shall load provider modules from managed .dll assemblies at runtime.

- Provider modules shall be built against a shared interface assembly.

- The Sequencer shall interact only with the shared interfaces.

- The Sequencer shall not reference hardware-specific provider projects directly.

- Each provider module shall expose one or more discoverable plugin classes.

- Plugin classes shall implement a required interface, for example:

public interface ISequencerProvider
{
    string ProviderName { get; }
    string ProviderVersion { get; }

    Task InitializeAsync(IProviderContext context);
    IReadOnlyList<IPoint> GetPoints();
    Task ShutdownAsync();
}

- The Sequencer shall discover valid provider classes using reflection.

- The Sequencer shall instantiate provider classes dynamically.

- Provider modules shall expose named points through the shared point interface.

- The Sequencer shall access hardware only through point interfaces.

- Provider modules shall own all hardware-specific logic, including protocols, register maps, addresses, scaling, and connection handling.

- The Sequencer core shall contain no SigCore, EZ-ZONE, Modbus, or device-specific logic.

- Provider modules shall declare their compatible interface version.

- The Sequencer shall reject incompatible provider modules.

- Provider modules shall report initialization, connection, read, write, and shutdown errors in a standard way.

- Provider modules shall be unloadable when practical through AssemblyLoadContext.

- Provider configuration shall be supplied at runtime by the Sequencer.

- Provider modules shall not require recompiling the Sequencer.

- A failed provider shall not crash the Sequencer.